Hardware-Software Co-Design for LLM Quantization:

A Comparative Analysis of ASIC and FPGA Approaches

Gyanu Pandey

CECS 528: Advanced Computer Architecture

California State University, Long Beach

I. INTRODUCTION

Large Language Models have fundamentally changed what artificial intelligence can accomplish. Models like GPT-4 and LLaMA demonstrate remarkable abilities in text generation, code synthesis, and complex reasoning. However, deploying these models at scale presents a significant engineering challenge. A model like LLaMA-70B requires approximately 140 GB just to store its weights, which exceeds the memory capacity of most available hardware. Even high-end datacenter GPUs like the NVIDIA A100, priced at over $15,000, provide only 80 GB of memory.

Quantization offers a practical solution by reducing numerical precision from 32-bit floating point to 8-bit or 4-bit integers, shrinking memory requirements by 4-8x. However, software-based quantization on standard GPUs introduces overhead from runtime dequantization, sometimes making inference slower rather than faster. This paper examines whether custom hardware designed specifically for quantized inference can overcome these limitations.

II. RELATED WORK

Software quantization tools like bitsandbytes, ONNX Runtime, and llama.cpp have made quantized inference accessible on commodity hardware. These libraries store weights in reduced precision but must convert them back to floating point for GPU computation. CipherCore benchmarks reveal mixed results: bitsandbytes runs 35% slower than unquantized baselines on certain workloads, while ONNX Runtime achieves a 1.7x speedup in optimal conditions.

Oltron [1], developed at Peking University and published at DAC 2024, introduces a custom ASIC that addresses the outlier problem in neural network quantization. Approximately 1% of weight values are outliers, 10-100x larger than typical values. Oltron handles this through adaptive quantization that assigns different precision levels per channel, combined with reconfigurable processing elements that switch between INT4 and FP16 operations at runtime.

FlightLLM [2], from Tsinghua University and published at FPGA 2024, uses reconfigurable FPGA fabric. Their architecture exploits sparsity through configurable DSP units that skip zero-value computations entirely, and keeps the key-value cache in fast on-chip SRAM rather than slower DRAM, eliminating a major memory bottleneck during autoregressive decoding.

III. METHODOLOGY

This study conducts a comparative analysis of the two hardware approaches against software-only baselines. The evaluation focuses on three metrics: inference throughput (tokens per second), energy efficiency (performance per watt), and accuracy preservation (perplexity degradation). Both papers evaluate on LLaMA variants, providing a reasonable basis for comparison. The analysis examines each system's architectural innovations and their impact on the memory bottleneck that dominates LLM inference.

IV. RESULTS AND DISCUSSION

Oltron achieves a 1.9x speedup over prior accelerators while maintaining less than 1% accuracy degradation. The chip, fabricated in 12nm technology, demonstrates 1.6x better energy efficiency through its tile-based dataflow that maximizes data reuse. The adaptive quantization scheme proves effective at handling outliers without the runtime dequantization overhead that plagues software solutions.

FlightLLM delivers even more dramatic improvements in energy efficiency, achieving 6x better performance per watt compared to GPU baselines. Running on a Xilinx U280 FPGA that draws only 75 watts, it matches or exceeds the throughput of an NVIDIA A100 consuming 300 watts. At datacenter scale where power and cooling dominate operational costs, this efficiency advantage translates directly to reduced total cost of ownership.

The results confirm that hardware-software co-design substantially outperforms software-only approaches. While the best software tools achieve 1.7x speedup, dedicated hardware consistently delivers 1.9x or better with far superior energy efficiency. The key advantage is eliminating the dequantization overhead entirely by computing directly on quantized values.

V. CONCLUSION

This comparative analysis demonstrates that custom hardware designed for quantized LLM inference significantly outperforms software implementations on general-purpose GPUs. ASICs like Oltron offer maximum performance through fixed-function optimization, while FPGAs like FlightLLM provide flexibility to adapt as models evolve. Both approaches achieve their gains by addressing the fundamental memory bottleneck through co-designed algorithms and architectures. As LLMs continue to grow in size and deployment scale, hardware-software co-design will likely become essential for practical inference systems.

REFERENCES

[1] Y. Liu et al., "Oltron: An ASIC for Efficient LLM Inference via Mixed-Precision Quantization," Proc. Design Automation Conference (DAC), 2024.

[2] S. Zeng et al., "FlightLLM: Efficient Large Language Model Inference with a Complete Mapping Flow on FPGAs," Proc. ACM/SIGDA International Symposium on FPGAs, 2024.

[3] T. Dettmers et al., "bitsandbytes: 8-bit Matrix Multiplication for Transformers at Scale," NeurIPS, 2022.

[4] CipherCore, "LLM Quantization Benchmark: Software Tools Comparison," Technical Report, 2024.